Lab– Microcontrollers

Objectives:

-To learn about microcontrollers and how they are used with sensors and output devices

-To learn about the Arduino Board and Arduino IDE

-To learn some basic programming in Arduino C language

Introduction

Arduino is an electronics platform based on easy-to-use hardware and software. Arduino boards are able to read inputs - light on a sensor, a finger on a button, or a Twitter message - and turn it into an output - activating a motor, turning on an LED, publishing something online. You can tell your board what to do by sending a set of instructions to the microcontroller on the board.  The brain of an Arduino is a microcontroller chip.

Equipment:

One Arduino, 1 LED, 330 Ohm resistor, 1 Servo motor, 1 low power Laser, 1 solar cell with clips, jumper wires, breadboard.

-Software - Arduino IDE

The Arduino integrated development environment is a software application that you can run from almost any computer. It allows you to write, test and run a set of instructions (programming code) that controls the electronic components on the Arduino board and any external components connected to it. Once this code is written on the IDE it can be uploaded and stored via a USB connection to the microcontroller chip.

The software can be downloaded from the following site:

https://www.arduino.cc/en/software

If it hasn’t been done for you already by your instructor, install the Arduino software and open up the Arduino IDE by clicking on the Arduino App icon on the desktop.

Note: If you get an error when uploading your program to Arduino such as “Error compiling for board Arduino Mega or Mega 2560” you might need to install the u8glib graphical library in order to successfully compile your program. Ask instructor for help.

On the main menu of the IDE go to File>>Examples>01.Basics>BareMinimum. This will open up the file below. This file is a template that contains two functions that are used in the majority of programs that you will write, void setup() and void loop(). These will be explained later in the document.

Figure 1 – Code on Arduino IDE startup

Before any code can be uploaded to the microcontroller on the Arduino a connection needs to be made between the hardware and software so that they are able to “talk” to each other. To do this, first connect the supplied USB cable from the computer to the Arduino board. Next you need to check that the communication port on the Arduino is connected to the communication port on the computer. On the main menu of Arduino IDE go to Tools>>Port and check and see that one of the ports is labeled “(Arduino Uno)”. Also make sure that in Tools>>Board that the “Arduino Uno” has been selected. If this is the case, then a connection has been established between the two devices.

Figure 2 - A Closer look at the Arduino Board

Activity 1a – Lighting up an LED

On the edge of the Arduino board you’ll see pins that are labelled from 0-13 (Digital) and ones that are labeled A0-A5(Analog In). These pins allow you to interact with the Arduino itself or any devices connected to it. You send signals (Analog or Digital) to the Arduino or pull signals from it via these pins. In this activity a simple circuit consisting of an LED and resistor will be connected to the Arduino and a short program will be written that tells the Arduino what to do with the circuit.

1.Wire up the circuit below and then connect a jumper wire from the resistor to digital pin 13 on the Arduino and from the LED to the ground pin (GND) on the board.

Figure 3 – Circuit on breadboard

2. Write the following code in the BareMinimum file you opened up earlier.

To control the Arduino and anything connected to it you need to write a program. The program contains a series of instructions telling the Arduino board what to turn on or off. The structure of an Arduino program is shown above. The void setup() and void loop () are the main functions in the code. They contain a series of instructions written by the user that are run as soon as he/she uploads the program into the Arduino. The setup () function executes all of the code inside of the curly braces { }. This code is executed once, when the Arduino is first turned on or when it is reset.

The loop () function contains the code that you want to have repeated over and over again. As long as the Arduino is running, the code inside of the curly braces will keep repeating, after the code in setup has run once.

pinMode () is a pre-existing function that can be called by setup() or loop () and  instructs the Arduino to use a pin number on the board to act in a particular way. In the code above, 13 is the number of the pin we will set to OUTPUT.

The delay() is another pre-existing function built into the Arduino IDE that tells the program to pause execution for a certain period of time before continuing on to the next statemen in the codet. The delay time in parenthesis is expressed in milliseconds.

digitalWrite() is a pre-existing function that tells the Arduino to set the voltage level to LOW (0 Volts) or High (5 Volts) on a particular pin on the board. In the code above, a high or low voltage will be sent to pin 13.

Semicolons in the code serve the same purpose as periods in English; they denote that we have reached the end of the line.

Activity 1b –Using variables in your program

Notice that in the program above you need to tell the functions (pinMode and delay) what pin number on the Arduino the LED is connected to and how long you want the LED to be on or off for. These numbers need to be written in the program several times and makes it inconvenient if you decide to change the delay time or the pinMode number at some point. A better way to write this program is with a variable.

To use a variable in a program you need to name it, declare the type of variable it is (integer, character, string, etc.) and assign a value to it.

Lines 2 and 3 in the program declare two variables of type int. This tells the program that these variables are integers. So, in line 3, for example, the variable ‘delayTime’ is an integer type and it has been assigned a value of 200. If you wanted to change how fast the LED is flashing, you will need to change this one value at the top of the program and it will be changed everywhere else that ‘delayTime’ is being used.

Rename this program, save it and then upload it to the Arduino.

Activity 1c – Control Statements For Loops

The program above causes the LED to continually flash on and off indefinitely. But what if you want to the LED to flash on and off every 200 milliseconds five times and then pause for 1 second (1000 milliseconds)? For this type of behavior you need to use a loop. A ‘For Loop’ is used in a program whenever you want to repeat a statement or series statements a fixed number of times.

Line 10 shows the syntax used when writing a for Loop. The term in parenthesis after the for statement and before the first semi-colon declares a variable j of type integer.

This variable will be used to by the program to determine how many times the statements inside the brackets { } need to be executed. In this case, it tells the program to run the loop 5 times. The expression after the second semi-colon (j=j+1) tells the program to increment the variable j by 1 every time it runs through the loop.

Rename this program, save it and then upload it to the Arduino.

Activity 1d – Conditional Statements if()

Connect a 2nd LED and resistor to your breadboard like you did above only now connect the LED to pin 12 of the Arduino Uno and the 2nd resistor to ground.

In the previous activity you wrote a program that controlled the number of times that the LED flashed. What if it is necessary to have the 2nd LED flash on and off the after the first LED has flashed a certain number of times.  For example, say we want to have the 2nd LED flash on and off on the condition that  the first LED has flashed on and off twice already. To accomplish this we need to use a conditional statement.

An if() statement is a conditional statement that allows you to make something happen or not, depending on whether a given condition is true or not. Modify your previous program to the one shown below.

In this program, we’ve added a few more lines. Line 2 declares another variable called pinNumber2 and it is assigned the value 12.

In the setup() function, line 8 adds another pinMode statement. This statement tells the Arduino that something will connect to digital pin 12 and that this pin will output a digital voltage.

The conditional statement starts on line 18. The if statement states that if the variable ‘j’ used in the For Loop is equivalent to 2 then the code in the curly brackets must be executed. Note the use of the double equals ‘==’ to state equivalence. The single equals is used to assign a number to a variable. This code will turn on the 2nd LED on your breadboard and then go off. The second LED should only come on when j equals 2.

Rename this program, save it and then upload it to the Arduino.

Activity 2 – Controlling a Servo Motor with Arduino

In a previous lab you built a circuit consisting of a motor and a sensor for electronic feedback control. Most motors are used to continually spin an object at a certain speed.

In this activity you will be working with a servo motor. This type of motor is used to rotate an object by a certain amount. They can be found in positioning systems used in robotics, industrial automation, and toys.

Figure 4 – Servo motor

Servos have three wires coming out of them. Out of which two will be used for Supply (positive and negative) and one will be used for the signal that is to be sent from the microcontroller unit. The red wire on the micro-servo supplied to you will be connected to a voltage source while the brown cable will be connected to ground. The orange cable is used to control the servo with an electrical pulse.

A servo motor uses an electrical pulse of variable width or PWM (Pulse width modulation) to position the motor shaft. The angle of rotation is controlled by the duration of the applied pulse to its Control PIN. The length of the pulse will determine how far the motor turns.

Figure 5 – Electrical pulse of variable width

- Connect the red wire of your servo to the pin labeled 5V on the Arduino Uno board in pin section labeled ‘Power’. The brown wire is connected to ground and the orange control wire needs to be connected to pin 9.

Type the following code in the Arduino IDE

Line 1 loads a package that is supplied with the Arduino IDE that contains a library of functions that can be used to control a servo motor. The #include<Servo.h> command is used to load the library.

Lines 2 and 3 declare and initialize two variables.

A servo motor is an object that the Arduino can interact with. You could connect multiple servos to the Arduino and those would be other objects that the Arduino can interact with. In the program, we keep track of all these objects by creating virtual objects and giving each one a name.

Line 4 creates a Servo virtual object called myServo and will be used to identify the servomotor at your station.

Line 7 will start the serial monitor. The serial monitor is used to display information from the program or to allow the user to enter data needed by the Arduino.

Line 8 tells the program that the myServo object which identifies your servo is attached to pin 9 of the Arduino.

In the loop function, line 13 is used to prompt the user to enter the angular position of the servo. Line 16 is a while loop that breaks the execution of the program and waits for the user to enter the specified information (the angular position of the servo in this case) before it continues.

Line 18 stores the information gathered from the user into a variable and line 19 sends that information to the Arduino through the control line to position the servo motor.

Mini-Project - Laser Communication – Transmitter and Receiver

In this activity you will build a transmitter for a laser communication system. Connect pin 13 of the Arduino to the red wire on the laser and the blue wire on the laser to the ground pin. Check to see if the message being transmitted by the laser is correct.

Transmitter Code being sent out by a laser pointer

Receiver Code using a solar panel or LDR

These images were created with Fritzing (https://fritzing.org)